Table of Contents

1  Introduction	1

1.1	Using the operating manual	1

1.2	Warning symbols	2

1.3	Intended use of the Front End Loader	2

1.4	Description of the Front End Loader	3

1.5	Noise	3

1.6	Transportation	3

2	Safety instructions	4

2.1	Before work.	4

2.2	Driving on roads	5

2.3	When working	6

2.4	After work	9

2.5	Maintenance	10

2.6	Safety stickers - meaning	11

3	Front End Loader	12

3.1  Preparations for use	12

3.2  Operation of the Front End Loader	13

3.3  Attachment and removal of the Front End Loader	14

3.4  Swinging the support legs in and out	17

3.5  Attachment and removal of the implements	18

3.6  Level display (Optional)	19

4 Implements	20

5 Maintenance	21

5.1  Hydraulic system	21

5.2  Bearing points	22

5.3  Threaded connections Danger .	22

5.4  Trouble shooting	23

6  Appendix	25

6.1  Circuit diagram - hydraulic system	25

6.2  Valve hookup（Single handle double-valve, Optional）	25

6.3  Technical data	27

7	Spare Parts	29

AppendixⅠ List of easily damaged parts ( Example)	29

Appendix II  Packing List ( Example)	30

- 1  Introduction

Dear customer,

We thank you for purchasing a Front End Loader from our company.

To ensure long-term satisfaction with this product, please read this manual carefully and familiarize yourself with its contents.

Thank you.

  - Using the operating manual

|  | Danger Read the entire operating manual before using the Front End Loader. |
| --- | --- |

Operation and service of the Front End Loader are described in this operating manual.

If you have questions concerning the operating manual, contact your dealer.

An operating manual is supplied with each machine. It is a component of the Front End Loader and must be kept in the tractor so that the driver or service personnel have immediate access to it for review purposes, if necessary.

Obtain a new operating manual immediately if the old manual is damaged or lost.

This manual contains information concerning the state of the technology that was valid at the time of delivery.

We reserve the right to make changes in the design and specifications of the loader and its components in the course of further technical development.

Direction information:

Left or right direction mentioned in this manual is seen from the driver.

  - Warning symbols

Warnings are used in this manual to protect you from injury and to protect the machine from damage. Always read and comply with these warnings. Warnings are shown in bold font.

Warnings are introduced with the words ....Danger .. or ....Caution .

The two warning levels have the following meaning:

Danger .

If the warning is ignored, there is danger of death, severe injury or significant machine damage.

Caution

If the warning is ignored there is danger of injury or machine damage.

Supplementally comments begin with the word.

Comment

These comments facilitate understanding or execution of a certain procedure.

  - Intended use of the Front End Loader

The Front End Loader has been built exclusively for agricultural and forestry loading tasks.

It should only be used with the implements provided by “SAMTRA”.

It should only be attached to those tractors for which “SAMTRA” designed it to be attached, or for which it is offered by “SAMTRA”.

Danger .

Any use extending beyond intended use is non-intended use. The manufacturer, “SAMTRA”, assumes no liability for damage resulting from non-intended use. This risk is borne solely by the user.

Intended use also includes compliance with the operating and service instructions prescribed by the manufacturer.

The Front End Loader should only be used and maintained by persons who are familiar with the contents of the operating manual through appropriate reading and instruction, and who have been particularly instructed concerning the dangers associated with operation of the Front End Loader.

  - Description of the Front End Loader

The Front End Loader is a loading device that is mounted on a tractor. It is used for lifting and moving loads.

The Front End Loader and the implements are activated via hydraulic cylinders.

The Front End Loader is powered by the tractor's hydraulic system, and is controlled from the driver's seat.

The support legs ensure that the detached Front End Loader is stable.

The implements are connected to the Front End Loader via the change frame. The change frame enables fast and safe implement change.

The Front End Loader consists of:

1. Implement cylinder

2. Change frame

3. Implement (e.g. bucket)

4. Support legs

5. Column

6. Lifting cylinders

7. Hydraulic pipes

8. Lifting arm (Parts that are permanently mounted on the tractor)

  - Noise

The noise of the front end loader is depending on the matching tractor.

  - Transportation

The machine is wrapped by the iron frame. So the forklift can move it easily.

- Safety instructions

  - Before work.

Danger.

- Before work check the tractor and the Front End Loader:

- Ensure that all components are firmly attached.

- Repair or replace damaged, excessively worn, or missing parts.

- Make all the required adjustments on the tractor and on the Front End Loader.

- Ensure that all components are lubricated regularly (sec. 5.2).

Caution.

If equipment is installed on the tractor that could collide with the Front End Loader or the implement, then remove this equipment before starting the load work.

Danger.

Arrest the hydraulics when driving on the road, or parking the tractor.

Front End Loaders should only be attached to tractors that have lockable hydraulics, if this is not the case then a stop valve must be installed.

Danger.

Tractor and Front End Loader work with hydraulic oil that is under high pressure in operation.

Check all hydraulic components and keep them in good condition.

Ensure that hydraulic components - particularly hoses - cannot be damaged by moving parts.

Caution .

If you use the Front End Loader at low outdoor temperatures, bring the hydraulic system up to working temperature beforehand. To do this, completely extend and retract all hydraulic cylinders several times.

Danger .

Ensure that the front tyres are inflated to the prescribed pressure for load operation as specified in the operating manual of the tractor.

Danger .

If the tractor has a seat belt, fasten the seat belt when driving.

  - Driving on roads

Danger. Ensure that the lights and reflectors of the tractor are not covered by the Front End Loader when driving on public roads. Danger. Do not drive on public roads with loaded implement. Danger. The implement must be at least 2 meters above the road surface. Comply with country-specific guidelines for positioning the Front End Loader when driving on roads. The horizontal distance between the front end of the Front End Loader and the middle of the steering wheel should not be more than 3.5 m. It may be necessary to remove the implement. Danger. If necessary, reduce the lift height when driving under bridges or overhead lines. Danger. Consider the greater length and higher weight of the tractor when driving on curves and for braking maneuvers. Danger. The maximum speed with Front End Loader and attached implement is 25 km/h for road driving and 10 km/h for load work.

  - When working

|  | Danger Familiarize yourself with the work area and the terrain. Keep a safe distance away from electrical lines. Only execute Front End Loader work if the visibility conditions are adequate. Ensure that the work area is adequately illuminated. Keep the cab windshield clean. |
| --- | --- |
|  | Danger Do not position yourself in the vicinity of raised Front End Loaders or in the vicinity of an unsecured load. Ensure that no one is positioned in the danger zone of the Front End Loader. |
|  | Danger Constantly observe your work area. Ensure that people or animals do not obstruct the work area. |
|  | Danger Only control the machine from the driver's seat |

|  | Danger. Do not lift or carry persons |
| --- | --- |
|  | Danger. Use extreme caution when working on a slope. Do not drive transverse to the slope with a raised load. The danger exists that you and the tractor will tip over. Push the load material together downhill. Scoop up the material in the depression at the bottom of the incline. Extend the wheel track of the tractor. |
|  | Danger. When driving on curves reduce the speed and lower the load. Danger. Do not drive the tractor in a jerky manner if the Front End Loader is in the highest load position and fully loaded Danger. Check the implement tilt when lifting the Front End Loader. Raised loads can fall on you if you scoop the implement too far. |

|  | Danger. Never stack multiple loads (bales, pallets) on top of each other. The upper loads can fall on you. Special implements, such as baling implements or pallet forks are designed in such a manner that they prevent loads from falling onto the tractor. |
| --- | --- |
|  | Danger. Check the implement tilt when lifting the Front End Loader. Raised loads can fall on you if you scoop the implement too far. |
|  | Caution. Drive straight into the load material. Do not execute any steering movement in this process. |
|  | Caution. Switch off the front axle suspension of the tractor. |

  - After work

|  | Danger. The following applies when you leave the driver seat: - Lower the Front End Loader onto the ground. - Engage the parking brake. - Move the gear shift into park position - Turn off the engine. Remove the key |
| --- | --- |

Danger

Comply with the following when removing the Front End Loader:

Only park the Front End Loader on a stable substrate and with attached implement.

Ensure that the support legs are securely positioned.

Only the operator should attach and remove the Front End Loader and the implements.

  - Maintenance

|  | Danger. Before starting maintenance work, put on your protective equipment (protective overalls, gloves, protective goggles, safety footwear) Danger. Lower the Front End Loader onto the ground. Depressurize the hydraulics and the hydraulic connections. Leave the Front End Loader coupled to the tractor. Let the machine temperature cool to below 55°C. Ensure good lighting. |
| --- | --- |
|  | Danger. NEVER stand between the front of the tractor and the transverse pipe of the Front End Loader. Never repair the Front End Loader with raised Front End Loader. Lower the Front End Loader onto the ground. |
|  | Danger. Never search for leaks with your fingers. Use suitable aids (a piece of wood or cardboard). Hydraulic oil escaping under high pressure can penetrate the skin and cause severe injury. If injured, consult a physician immediately. |

  - Safety stickers - meaning

If a sticker should get damaged or lost, obtain a replacement without delay.

|  | Danger. Only park the Front End Loader with attached implement. |
| --- | --- |
|  | Danger. Ensure that no one is positioned in the work area of the Front End Loader |
|  | Procedure for closing and disconnecting the HYDROFIX hydraulic quick coupler. |

- Front End Loader

Comply with the safety instructions in section 2 for all activities with the Front End Loader.

- 3.1 Preparations for use

The Front End Loader is delivered completely assembled and ready for connection.

Danger

Hydraulic lines and threaded connections that hydraulically connect the Front End Loader and tractor are loosely pre-mounted in the factory.

After final routing of the lines firmly tighten all threaded connections. After 5 operating hours retighten all threaded unions.

The required tightening torques is specified in section 5.3.

Caution

If equipment is installed on the tractor that could collide with the Front End Loader or the implement, then remove this equipment.

- 3.2 Operation of the Front End Loader

|  | A…Front End Loader-lifting B…Front End Loader-lowering C…Implement - scooping D…Implement - dumping S…Front End Loader-float position |
| --- | --- |
|  | Lifting: Pull the operating lever back to lift the Front End Loader (A). |
|  | Lowering: Move the operating lever forward to lower the Front End Loader (B). |
|  | Scooping: Move the operating lever to the left to scoop the implement (C) |
|  | Dumping: Move the operating lever to the right to dump the implement (D). |

|  | Float position: If you push the operating lever all the way back, the operating lever will lock in float position (S). In float position the Front End Loader lowers to ground level. When driving with float position the Front End Loader adjusts to ground level. |
| --- | --- |
|  | Deactivate float position by pulling the operating lever back into neutral position. |

Danger

When elevate the Front End Loader, please ensure that the angle of bucket bottom with the ground is more than or equal to 0 degree (As shown in the following figure, angle G), otherwise it might damage the Front End Loader

3.2.1 Operation via tractor hydraulics

The Front End Loader is operated via two supplemental hydraulic control devices of the tractor.

Four hydraulic lines connect the tractor hydraulics directly with the Front End Loader.

Almost all tractors with supplemental electro-hydraulic control units have a joystick located to the right of the driver seat. This joystick enables convenient loader guidance.

Installation is described in the tractor-specific installation instructions.

3.2.2 Operation via joystick

The tractor is equipped with a mechanical control block on the add-on part.

There is a joystick in the tractor cab.

The joystick is connected to the control block via an electrical cable.

The joystick enables easy movement and fatigue-free loader guidance.

3.2.3 Operation via 3rd control circuit

The 3rd control circuit is an optional supplemental function for all Front End Loader types.

The 3rd control circuits are additional double-action oil circuits on the Front End Loader. With these circuits you control the additional hydraulic functions of the implement.

- 3.3 Attachment and removal of the Front End Loader

The Front End Loader is connected to the add-on parts bolted onto the tractor via a wedge locking mechanism. It takes one person about 2 to 4 minutes to attach or remove the Front End Loader.

Attachment:

Only the operator should attach and detach the Front End Loader.

The sticker shown below is affixed to the outside of the left column.

Replace the sticker if it is damaged or lost.

1. Loosen the clamping lever. and Move tractor forward.

2. Depressurize the hydraulic system and connect hydraulic lines.

3. Scoop the bucket by operating the joystick. Until the Front End Loader can enter the add-on parts.

4. Unscoop the bucket by operating the joystick. Until the Front End Loader can lowed the hole of the add-on parts.

6. Put the Front End Loader and the add-on parts together by the pins.

Removal:

|  | Danger . Only park the Front End Loader with implement attached and on a stable substrate. Otherwise the Front End Loader could tip over. Ensure that you park on a stable substrate |
| --- | --- |
| 1. Engage the parking brake and lower the Front End Loader onto the ground. 2. Take out the pins 3. Swing the support legs downward, and plug into the pins to the support legs. 4. Lower the Front End Loader further until the rear locking pins rest on the slide rails. 5. Uncouple the hydraulic lines 6. Drive the tractor in reverse out of the Front End Loader. |  |

- 3.4 Swinging the support legs in and out

|  | Swinging out: 1... Arrest(adjustable) 2…Support leg 3…Support foot 4…small pin 5 big pin Danger . Ensure that you park on a stable substrate. |
| --- | --- |
|  | Lower the Front End Loader onto the ground without pressure. Take out the small pin from A, Swing the support leg down until it reaches the ground. |
|  | Adjust the arrest. Infix the big pin into C |
|  | Swinging in: Take out the big pin, and pull the support leg in the direction indicated by the arrow D. Adjust the arrest to the position A, Lock the arrest and the support led together with the small pin. |

- 3.5 Attachment and removal of the implements

Only the operator should attach and remove the implements.

Attachment

1. Turn the handle to the right, so that the implement mounted on the hooks of the of the change frame.

2. Turn the handle to the left. The implement is now firmly connected with the Front End Loader.

Danger.

During the locking process, the handle should turn enough, so that it can ‘t be back .

Removal

Implements are removed in the reverse sequence.

- 3.6 Level display (Optional)

Check the horizontal implement position via the level indicator.

Place the implement in a horizontal position. Loosen the handle bolt (2). Change the position of the pipe (3) so that the pipe and the rod (1) are flush on curve. Re-tighten the handle screw.

3.7 Mechanical parallel guide

When lifting and lowering the Front End Loader the implement is guided parallel through the guide linkage. This keeps the incline of the implement constant when lifting and lowering.

The mechanical parallel guide is helpful for all load tasks. This function is particularly important when loading pallets and for stacking bales.

Comment

The parallel guidance is particularly effective with an implement that is horizontal or scooped. It does not function with an implement that is tilted out.

Check the horizontal implement position via the level indicator.

- 4  Implements

The operating manual for the different implements is not a component of this operating manual.

Danger.

Only use implements specified by “SAMTRA”.

Incorrectly designed implements from other manufacturers can damage the Front End Loader.

Danger

Only use implement sizes that are approved for the size of your Front End Loader. Otherwise tractor, Front End Loader, or implement can be damaged.

Ask your dealer about the suitable implement size.

- 5  Maintenance

For maintenance work follow the instructions provided in the safety information in section 2.7.

The owner or the user of the Front End Loader is responsible for regular maintenance.

Clean the Front End Loader before performing maintenance tasks. This applies particularly when maintaining the hydraulic system.

Danger.

Only use spare parts that are approved by the manufacturer.

The maintenance intervals are specified in effective operating hours of the tractor with Front End Loader. The maintenance intervals apply for normal work conditions. Shorten the maintenance intervals if there are difficult work conditions.

- 5.1 Hydraulic system

The instructions and guidelines provided for the tractor hydraulics apply for maintenance of the hydraulic system.

Danger.

Ensure cleanliness when working on the hydraulic system. Clean hydraulic oil is required for trouble-free function.

Danger.

Check the hydraulic oil level of the tractor on level ground and with lowered implement.

Only use oil as specified by the operating manual of the tractor.

Danger.

Check hydraulic lines and connections for wear and leaks.

Replace worn or leaky hydraulic lines.

Danger.

Repair or replace leaky hydraulic cylinders.

You will need special tools to repair the hydraulic cylinders.

Have defective cylinders repaired by your authorized dealer.

- 5.2 Bearing points

|  | Caution. Lubricate all bearing points after every 20 operating hours. Also lubricate the bearing points on the implements. Prior to lubricating, offload the bearing points by lowering the implement to the ground. |
| --- | --- |
| Lift the Front End Loader slightly so that you can lubricate the interior lubricating nipple In this process, place the Front End Loader on the implement tip. |  |

- 5.3 Threaded connections Danger .

Re-tighten all threaded connections after 5 operating hours.

Danger.

Check the threaded connections for firm seat every additional 100 operating hours. Re-tighten threaded connections as needed.

| Screw tightening torque |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Screw | Class |  |  | Screw | Class |  |
|  | 8.8 | 10.9 |  |  | 8.8 | 10.9 |
|  | Nm | Nm |  |  | Nm | Nm |
| M8 | 23 | 33 |  | M20 | 380 | 530 |
| M8x1 | 25 | 35 |  | M20x2 | 400 | 560 |
| M10 | 46 | 65 |  | M20x1.5 | 420 | 590 |
| M10x1.5 | 59 | 80 |  |  |  |  |
| M10x1.25 | 49 | 69 |  | M22 | 510 | 720 |
| M12 | 80 | 110 |  | M22x2 | 540 | 750 |
| M12x1.5 | 84 | 118 |  | M22x1.5 | 560 | 790 |
| M12x1.25 | 88 | 123 |  | M24 | 630 | 890 |
| M14 | 130 | 180 |  | M24x2 | 680 | 950 |
| M14x1.5 | 138 | 190 |  | M27 | 930 | 1310 |
| M16 | 190 | 270 |  | M27x2 | 995 | 1400 |
| M16x1.5 | 210 | 290 |  | M30 | 1260 | 1770 |
| M18 | 270 | 380 |  | M30x2 | 1370 | 1930 |
| M18x2 | 280 | 400 |  |  |  |  |
| M18x1,5 | 300 | 420 |  |  |  |  |
|  |  |  |  | 9/16-18 UNF | 24~28 |  |
|  |  |  |  | 3/4-16 UNF | 49~53 |  |

- 5.4 Trouble shooting

Faults on the Front End Loader are frequently caused by factors that are not due to a Front End Loader malfunction. Many problems can be avoided through regular maintenance.

If there are faults, please check the following first:

- Is there sufficient oil in the tractor's hydraulic oil tank?

- Are you using the right oil? Only use oil as specified by the operating manual of the tractor. The wrong oil can cause foaming and leaks.

- Is the hydraulic oil clean and free of moisture? Replace oil and filter if necessary.

- Are hoses correctly mounted? The connections must be locked in place.

- Are hoses and connections damaged, jammed, or twisted?

- Were the cylinders of the Front End Loader moved into their end positions several times to get the air out of lines and cylinders?

- Have you taken low outside temperature into account? Has the oil reached operating temperature?

If the measures described above are unsuccessful, the following table may help you to localize and correct the fault.

If you require support, contact your dealer.

| Fault description | Cause | Fault correction |
| --- | --- | --- |
| A) Low lifting and tearing forces | Insufficient oil pressure | Check the tractor hydraulics |
| B) Operating lever difficult to move. | Cables are difficult to move. | Check the routing and ease of movement of the cables. Oil or replace the cables. |
| C) Front End Loader and implement move too slowly or do not move at all. | 1) Insufficient oil in the hydraulic system. 2) Hydraulic couplings incorrectly connected. 3) Hydraulic coupling defective. 4) Oil flow too low. 5) Pressure control valve of the loader jams in open position. | Check oil level and top off if necessary. Check connections. Check couplings, replace if necessary. Check the tractor hydraulics. Check the pressure in the hydraulic system. |
| D) Front End Loader and/or implement work in the wrong direction relative to the operating lever. | 1) Hydraulic connection incorrectly Connected. 2) Cables incorrectly mounted. | Check the hydraulic connections, correct if necessary. Check the cable connection, correct if necessary. |
| E) Slow or uneven lifting of the Front End Loader. | 1) Insufficient oil in the hydraulic system. 2) Insufficient engine rpm. 3) Hydraulic fluid too cold. 4) Excessive load in the vehicle. 5) Hydraulic coupling defective. 6) Internal leakage in the hydraulic cylinder. 7) Pressure control valve set incorrectly. 8) Interior leakage in the control block. | Check oil level and top off if necessary. Increase engine rpm. Warm hydraulic system to working temperature. Decrease load. Check couplings, replace if necessary. Check the cylinders, repair or replace defective cylinders. Check the setting of the pressure control valve. Check control block, replace if necessary. |
| F) Insufficient lifting force | 1) Internal leakage in the hydraulic cylinder. 2) Excessive load in the vehicle. 3) Pressure control valve set incorrectly. 4) Interior leakage in the control block. | Check the cylinders, repair or replace defective cylinders. Decrease load. Check the setting of the pressure control valve. Check control block, replace if necessary. |
| G) Air in the hydraulic system. (Indicated by the foamy hydraulic fluid.) | 1) Hydraulic pump takes in air. 2) Hydraulic filter fouled. | Check the lines between hydraulic pump and tank for loose or defective connections. Check or replace hydraulic filter as necessary. |
| H) Leakage on the hydraulic couplings of the Front End Loader. | Leak due to penetrating grime. | Clean coupling, replace if necessary. If the Front End Loader are not used, seal the hydraulic couplings with the protective caps. |
| I) Front End Loader blocked during the lift or lowering movement. | 1) Coupling not completely closed. 2) Coupling defective. | Check the hydraulic coupling. Replace defective coupling halves. |
| J) Front End Loader shakes when lowering loads. | Lowering speed too high | Throttle lowering speed. |
| K) Implement cylinders extend, however they do not retract. | 1) Piston seal in the implement cylinder is defective so that the piston and ring surface are connected to each other. 2) Seat valve does not return to start position after rapid traverse is switched-on. | Check the cylinders separately from each other for leaks, replace defective cylinders. Remove the seat valve and examine for fouling, replace if necessary. |
| L) Leaks on the hydraulic block | 1) Threaded connections loose 2) Leakage between magnet and valve | Re-tighten threaded connections. Unscrew the knurled nut, remove magnet, re-tighten the magnetic core with open-jaw spanner. |

- 6  Appendix

- 6.1 Circuit diagram - hydraulic system

Assignment of the hydraulic lines

- 6.2 Valve hookup（Single handle double-valve, Optional）

Valve Parts Schedule

| Key | Part Number | Description | Quantity |
| --- | --- | --- | --- |
| 1 | 673974 | Hydraulic Hose | 1 |
| 2 | 673975 | Hydraulic Hose | 1 |
| 3 | 673976 | Hydraulic Hose | 1 |
| 4 | 673977 | Fittings | 4 |
| 5 | 673290 | Quick connector | 4 |
| 6 | 673293 | Fittings | 4 |
| 7 | 673993 | Fittings | 2 |
| 8 | 673742 | Fittings | 2 |
| 9 | 673743 | Fittings | 1 |
| 10 | 673744 | Hydraulic Hose | 1 |
| 11 | 673745 | Fittings | 1 |
| 12 | 673746 | Hydraulic Hose | 1 |
| 13 | 673747 | Fittings | 1 |
| 14 | 673748 | Hydraulic Hose | 1 |

- 6.3 Technical data

The technical data depends on the tractor type used and thus can vary from the specified values. The forces are specified for an operating pressure of 185 bars.

The payload is calculated as the lifting force minus implement weight.

| Loader Model |  | TZ02D | TZ03D | TZ04D | TZ06D | TZ08D | TZ10D | TZ12D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tractor model | hp | 15~25hp | 20~40hp | 30~55hp | 40~60hp | 50~75hp | 70~100hp | 90~130hp |
| Max. Lift Height at the attachment pivot point. (A) | mm | 2350 | 2552 | 2740 | 3010 | 3400 | 3710 | 3970 |
| Dump Height (C) | mm | 1730 | 1840 | 2100 | 2370 | 2480 | 2720 | 3000 |
| Dump Length (D) | mm | 370 | 500 | 600 | 640 | 690 | 740 | 900 |
| Height rocker fulcrum(E) | mm | 1120 | 1200 | 1360 | 1500 | 1630 | 1780 | 1930 |
| Tip in angle（F） | °degree | 48 | 48 | 48 | 48 | 54 | 54 | 54 |
| Tip out angle（G） | °degree | 52 | 58 | 58 | 58 | 58 | 58 | 58 |
| Scooping depth (H) | mm | 90 | 100 | 100 | 120 | 140 | 180 | 200 |
| Lifting force down in the implement fulcrum | N1 | 1821 | 2224 | 2144 | 2150 | 1997 | 2308 | 2403 |
|  | N2 | 1184 | 1474 | 1321 | 1286 | 1187 | 1452 | 1690 |
| Bucket width | mm | 1250 | 1250 | 1400 | 1500 | 1600 | 1700 | 2000 |
| Bucket capacity | m3 | 0.18 | 0.18 | 0.2 | 0.22 | 0.43 | 0.46 | 0.6 |
| Rated lift capacity | Kg | 200 | 200 | 300 | 400 | 600 | 800 | 1000 |

Note: Specification may vary based on the tractor used. No further informing will be given for any modification of the data.

# Spare Parts

| Content | QTY |
| --- | --- |
| Mainframe | One |
| List of easily damaged parts | One（such as Appendix I） |
| OWNERS MANUAL | One |
| Table of Fittings | One (such as Appendix II) |

AppendixⅠ List of easily damaged parts ( Example)

| No. | Standard | Name | Specification | Qt. |
| --- | --- | --- | --- | --- |
| 1 | JB982-77 | Washer | 14 | 1 |
| 2 | JB982-77 | Washer | 18 | 8 |
| 3 | JB/ZQ 4436-86 | Washer | 8 | 4 |
| 4 | GB 1235-76 | O-seal | 14×1.9 | 4 |
| 5 | GB 1235-76 | O-seal | 11×1.9 | 8 |
| 6 | GB 1235-76 | O-seal | 16×1.9 | 6 |

Appendix II  Packing List ( Example)

TZ08DPT-FT824 The list of the front end loader-with forks and buckets

| No. | Standard number | Name | Specification | QTY | Connect position |
| --- | --- | --- | --- | --- | --- |
| 1 | GB1235-76 | O-seal | 11×1.9 | 10 |  |
| 2 | GB1235-76 | O-seal | 14×1.9 | 10 |  |
| 3 | GB/T5783-1986 | bolt | M16×50 | 8 | Connect tractor and the add-on part |
| 4 | GB/T5783-1986 | bolt | M16×60 | 8 |  |
| 5 | GB/T97.1-1985 | Flat washer | 16 | 16 |  |
| 6 | GB/T 93-1987 | Spring washer | 16 | 16 |  |
| 7 | GB/T5783-1986 | bolt | M14×50 | 4 | Connect tractor and the add-on part |
| 8 | GB/T 93-1987 | Spring washer | 14 | 4 |  |
| 9 | GB/T97.1-1985 | Flat washer | 14 | 4 |  |
| 10 | GB/T5783-1986 | bolt | M12×50 | 4 | Connect the joint of the bottom and the add-on part |
| 11 | GB/T 93-1987 | Flat washer | 12 | 8 |  |
| 12 | GB/T97.1-1985 | Spring washer | 12 | 4 |  |
| 13 | GB/T6170-1986 | nut | M12 | 4 |  |
| 14 | GB10-88 | bolt | M16×55 | 10 | bucket |
| 15 | GB/T6170-1986 | nut | M16 | 10 |  |
| 16 | GB/T93-1987 | Spring washer | 16 | 10 |  |
| 17 | TZ08.1P-101 | Teeth of the bucket |  | 5 |  |
| 18 |  | specification |  | 1 |  |
| 19 |  | Scheme of installation |  | 1 |  |
| 20 |  | nylon strip | 8×500 | 4 |  |
| 21 | TZ04.7-102 | hose nipper | A23 | 4 | 18-NPT1/2 |

Note the Front End Loader data here.

There is a description of where the type plate of the Front End Loader is located.

| Front End loader |  |
| --- | --- |
| Front End loader model |  |
| Production number |  |
| Year of manufacture |  |
| Delivery list number |  |
| Purchase date |  |

Note the data here for the tractor to which the Front End Loader has been attached.

| Tractor |  |
| --- | --- |
| Tractor model |  |
| Production number |  |
| Year of manufacture |  |