OPERATION MANUAL

GRASS MOWER

CATALOGUE

DESCRIPTION	3

TO THE DEALER	3

TO THE OWNER	4

YOUR SAFETY IS INVOLVED, BE CAREFUL!!	4

GENERAL INFORMATION	4

TECHNICAL SPECIFICATIONS	5

SAFETY RULES & ACCIDENT PREVENTION	6

OPERATION	8

ASSEMBLY INSTRUCTIONS	9

ATTACHING TO THE IMPLEMENT	9

THE POWER TAKE OFF SHAFT (PTO)	9

DAILY PRE-OPERATION CHECKLIST	10

CUTTING HEIGHT ADJUSTMENT	11

REMOVING MOWER FROM THE TRACTOR	12

OWNER SERVICE	12

BLOCKING METHOD	13

LUBRICATION INFORMATION	14

CUTTER BAR AND BLADE SERVICING	14

GEARBOX SERVICE	15

GEARBOX MAINTENANCE	16

GEARBOX SPARE PARTS	18

SPARE PARTS FOR SG SERIES MOWER	19

SPARE PARTS LIST	19

TROUBLE SHOOTING	22

WARRANTY	23

WARRANTY AND SAFETY ACKNOWLEDGEMENT	24

OPERATOR INSTRUCTIONS AND PARTS LIST

Dear Customer,

The safe operation and maintenance of your Mower is very important to prevent any personal injury and / or equipment damage. Read and fully understand the following instructions before operating or carrying out maintenance. This instruction manual must be read in conjunction with the specific Tractor Operation Manual, Hazard Risk and Farm Machinery Safe Work Practices, prior to the equipment being placed in service.

We warrant new products sold but it to be free from defects in material and workmanship for a period of twelve months for farmers or six months for contractors, after delivery to the initial user. This warranty is for the replacement or repair of any part found to be defective during the warranty period. This warranty excludes any transport freight costs incurred by us in giving effect to the warranty and specifically excludes all normal maintenance costs. This warranty also excludes loss of income incurred by the owner due to down time. Replacement parts provided under the terms of this warranty are covered for the remainder of the warranty period applicable to the product in which they are installed as if such parts were original components of that product.

It is the dealer’s responsibility to deliver this unit to you fully assembled and to assist you in the installation. Before use carefully check the entire machine for transit damage and report any problems immediately. We reserves the right to make changes or improvements in design and construction at any time, without incurring the obligation to make these changes on previously manufactured units.

- DESCRIPTION

The SG Series Mower is a three point linkage tractor mounted implement designed for grass cutting. The SG Series Mower is available in three models which include the SG-1.2, SG-1.5 and SG-1.8. All models are covered in this manual.

Please ensure the unit you have purchased has the horse power rating to match that of your tractor.

Due to the very nature of this implement it is extremely important that you read and understand this owner’s manual, before commencing operation.

- TO THE DEALER

Assembly and proper installation of this product is the responsibility of the dealer. The dealer must complete all items on the Warranty Registration & Installation Form included in this manual before releasing the implement to the new owner. Both dealer and customer must sign the registration, which certifies that all Dealer Checklist items have been completed. The dealer must return a copy of the Warranty Registration & Installation Form to us.

- TO THE OWNER

Read this manual before operating your implement. The information presented will prepare you to do a much better and safer job. Keep this manual handy as a reference. Ensure all operators read this manual carefully and become acquainted with all the adjustments and operating procedures before attempting to operate. Keep a record that the operator has read and understands the correct operating procedures as outlined in this manual. The implement you have purchased has been carefully engineered and manufactured to provide dependable and satisfactory use. Like all mechanical products, it will require routine cleaning, upkeep and maintenance. Lubricate the implement as specified. Observe all safety information in this manual and safety decals on the implement.

- YOUR SAFETY IS INVOLVED, BE CAREFUL!!

Safety is a primary concern in the design and manufacture of our products. This implement like all mechanical products can cause serious injury if misused. Please ensure you read, understand and follow this manual for your own safety and that of others.

- GENERAL INFORMATION

The purpose of this manual is to assist you in operating and maintaining your Mower. Read it carefully. It provides information and instructions that will help you achieve years of dependable performance. These instructions have been compiled from extensive field experience and engineering data. Some information may be general in nature due to the unknown and varying operating conditions. However, through experience and these instructions, you should be able to develop procedures suitable to your particular situation. The illustrations and data used in this manual were current at the time of printing, but in an effort to bring you better products we are always implementing continuous improvements that may vary your implement slightly in detail. We reserve the right to redesign and change the machine as may be necessary without notice.

Some illustrations in this manual show the Mower with Safety Shields removed to provide a better view. The Mower should never be operated with any safety shielding removed.

Throughout this manual, references are made to right and left direction. These are determined by standing behind the equipment facing the direction of forward travel. Blade rotation is anti-clockwise as viewed from the top of the Deck.

- TECHNICAL SPECIFICATIONS

| Model |  | SL-1.2 | SL-1.5 | SL-1.8 |
| --- | --- | --- | --- | --- |
| Cutting Width (mm) |  | 1200 mm | 1500 mm | 1800 mm |
| Matched Power (hp) |  | 25~40 | 45~60 | 65~80 |
| Cutting Height (mm) |  | 25-100 | 25-100 | 25-100 |
| Rated PTO Speed (rpm) |  | 540 | 540 | 540 |
| Working Efficiency (m2/h) |  | 4200 | 5800 | 6900 |
| Blade Thickness (mm) |  | 10 | 10 | 10 |
| No. of Blades (pcs) |  | 2 | 2 | 2 |
| Cat. of 3-point Linkage |  | Ⅰ- type | Ⅱ- type | Ⅱ- type |
| Overall Dimensions.(mm) | Length | 2200 | 2340 | 2640 |
|  | Width | 1340 | 1640 | 1940 |
|  | Height | 900 | 1040 | 1040 |
| Weight (kg) |  | 230 | 290 | 350 |

Specifications are subject to alteration without notice.

- SAFETY RULES & ACCIDENT PREVENTION

ATTENTION, BE CAREFUL, YOUR SAFETY IS INVOLVED!

Safety is a primary concern in the design and manufacturing of our products. Unfortunately, our efforts to provide safe equipment can be wiped out by the single careless act of an operator. In addition to the design and configuration of equipment, hazard control and accident prevention are dependent upon the awareness, concern, prudence and proper training of personnel involved in the operation, transport, maintenance and storage of equipment.

Under no circumstances whatever shall the tractor/implement combination be operated whilst carrying a passenger either on or in the tractor or on the implement.

Safety instructions are important! Read all attachments and unit manuals; follow all safety rules and safety decal information. Failure to follow instructions can result in serious injury or death.

- If you do not understand any part of this manual and need assistance, your local dealer should be able to direct you on where you can receive appropriate training. No operator however experienced in farm machinery operation they may be, should attempt to use any piece of machinery that they have not been trained to use.

- Keep hands and body away from pressurized lines. Use paper or cardboard, not hands or other body parts, to check for leaks. Wear safety goggles. Hydraulic fluid (oil) under pressure can easily penetrate the skin and will cause serious injury or death.

- Make sure that all operating and service personnel know that if hydraulic fluid (oil) penetrates the skin, it must be surgically removed as soon as possible by a doctor familiar with this form of injury or gangrene, serious injury or death will result. Contact a physician immediately if fluid enters skin or eyes, do not delay.

- Know your controls and how to stop quickly in an emergency.

- Operators must be instructed in and be capable of the safe operation of the equipment and all controls. Do not allow anyone to operate this equipment without proper instruction.

- Do not allow children or untrained persons to operate equipment. Check that all hardware is tight and properly installed.

- Always wear relatively tight fitted clothing to avoid entanglement in moving parts. Wear heavy duty, rough-soled boots and protective equipment for eyes, hair, hands, hearing and head.

- Ensure implement is properly attached, adjusted and in a good condition so that it can operate safely. Make sure all spring activated locking pins or collars on the Power Take-Off (PTO) Shaft move freely, are well greased and are firmly seated in the tractor PTO splined angular groove.

- Before commencing operation, check all equipment driveline guards for damage and make sure they rotate freely. Replace any damaged guard or guards that do not rotate freely on drivelines.

- Tractors MUST be fitted with Roll-Over Protection Structure (ROPS) or ROPS Cap. It is also advisable that your tractor be fitted with a seatbelt, where fitted it must be worn at all times. Falling from your tractor can result in death from being run over or crushed. If your tractor is fitted with a folding ROPS, keep it in the “locked up” position at all times.

- Inspect chains, shackles, deflectors and all parts for wear or damage. Replace if damaged. Remove any debris that has accumulated on the implement, tractor or engine to avoid fire hazard.

- Ensure your tractor has a fire extinguisher and ensure you know how to use the extinguisher in the event of an emergency. Ensure implement is properly attached, adjusted and in good operating condition. Ensure all safety decals are installed. Replace if faded or damaged. Ensure shields and guards are properly installed and in good working condition. Replace if damaged.

- During routine checks and/or repairs ensure that no one can switch on the tractor or Implement accidentally. Keep the key to the tractor in your pocket.

- OPERATION

In addition to the design and configuration of equipment, hazard control and accident prevention are dependent upon the awareness, concern, prudence and proper training of personnel involved in the operation, transport, maintenance and storage of equipment. Remember it is the owner’s responsibility to ensure all operators are fully informed and trained.

“The best safety device is an informed careful operator”

Do not allow children or untrained persons to operate this implement.

Keep all people and animals away from the implement during start up, operating and when stopping. Never place any part of the body underneath the implement or between moveable parts even when engine has been turned off.

Follow Operator’s Manual instructions for working on your implement or have servicing carried out by a qualified dealer.

During routine checks and/or repairs ensure that no one can switch on the tractor or implement accidentally. Keep the key to the tractor in your pocket.

Make sure all spring activated locking pins or collars on the Power Take Off (PTO) Shaft move freely, are well greased and are firmly seated in the tractor PTO splined angular groove.

Inspect operating areas and remove any foreign objects that could be thrown, causing injury or damage. It is very important to always watch for foreign objects when you are operating and avoid them as they can cause injury or damage.

When operating along roads, pathways or populated areas fit approved chains or shielding (which is designed to reduce the possibility of objects being thrown) to the implement. If the implement is missing or has damaged shielding, operation must be stopped when anyone comes within 100 meters.

Stop implement, then tractor, immediately upon striking an obstruction. Turn off engine, remove key, inspect and repair any damage before resuming operations. Always block implement when inspecting. Always wear relatively tight fitted clothing to avoid entanglement in moving parts.

- ASSEMBLY INSTRUCTIONS

Depending on the location of the dealer, the implement may be shipped partly assembled or in parts. In any case, the assembly of this implement is the responsibility of the dealer. It should be delivered to the new owner completely assembled, lubricated and adjusted for normal cutting conditions. Make sure all spring activated locking pins or collars on the Power Take Off (PTO) Shaft move freely, are well greased and are firmly seated in the tractor PTO splined angular groove. Firmly tighten all nuts, bolts, screws and shackles before operating. Do not operate the implement unless it has chains or rubber shielding in place.

- ATTACHING TO THE IMPLEMENT

Always connect the tractor to the implement on a level surface and never stand in between the tractor and the implement.

Always adjust the tractor lower linkage arms anti-sway devices to prevent implement from swinging from side to side during transport and operation. Failure to do so may result in serious injury or damage. Damage like bent and twisted implement towers are consumable items and are NOT covered by warranty.

Line the tractor up so that the double clevis mounting point on the implement can be connected to the lower link arms of the tractor and secured with lynch pins. Attach tractor top link to implement top double clevis with top hitch pin and secure with lynch pin. Always ensure that the tractor connection (lower arms or top link) are not outside the double clevis mounting point.

- THE POWER TAKE OFF SHAFT (PTO)

PTO shafts come in standard lengths so that they can be adapted to all tractors. This means that you may have to shorten the PTO Shaft to suit your tractor. Hold the male and female sections parallel to each other and check for a minimum 15cm overlap. If there is less than the minimum overlap, do not use that shaft. Contact your dealer and you may need to purchase a special shaft for your situation. If it is too long, equal lengths must be cut off both the male and female shafts.

- DAILY PRE-OPERATION CHECKLIST

(Owners Responsibility)

- Review and follow safety rules outlined in this manual.

- Check the implement is properly and securely attached to the tractor.

- Make sure PTO spring activated locking pin is well greased, slides freely and is firmly seated in tractor PTO splined angular groove.

- Set tractor PTO at 540 rpm (Some Multi-Heads may have been designed for 1000rpm). Lubricate all grease fitting locations. Make sure PTO joints and shaft are well lubricated.

- Check all hardware (i.e. bolts, nuts, shackles, chains, etc) are properly installed, secured and in good condition.

- Check blades are sharp and secure and cutting edge is positioned to lead when rotating anti-clockwise.

- Check that all shields and guards are properly installed and in good condition.

- Check cutting height, front to rear attitude and top link adjustment.

- Ensure tractor PTO and transmissions are in neutral before starting engine.

- Inspect area that you will be operating in and remove any object that may cause injury or damage.

- Vibration tends to loosen bolts during operation. All hardware should be checked regularly. It is good practice to check implement before each operation to ensure all hardware is secure.

- CUTTING HEIGHT ADJUSTMENT

Avoid low cutting heights. Striking the ground with blades produces one of the most damaging shock loads a Mower can encounter. Allowing blades to repeatedly make contact with the ground will cause damage to the Mower and drivelines.

Keep all persons away from operators control area while performing adjustments, service or maintenance.

During routine checks and/or repairs ensure that no one can switch on the tractor or implement accidentally. Keep the key to the tractor in your pocket.

The Mower should be operated at the highest position that will give the desired cutting result. This will help prevent the blades striking the ground, reducing blade wear and undue strain on the implement. For best results under heavier cutting conditions, always tilt the implement so that the front is 25-50mm lower. This tilt decreases the horsepower requirements and increases potential ground speed. When fine cutting is desired, adjust the implement so the deck is level or slightly lower in the rear. This will keep the foliage under the cutter until thoroughly shredded. Operating in this manner will require more horsepower and a slower ground speed to get the desired result.

Never place any part of the body underneath the implement or between moveable parts even when engine has been turned off.

Before commencing any adjustment, maintenance, or cleaning. Always disengage the Power Take Off (PTO) Shaft, switch off the tractor engine, remove key and wait until all moving parts have come to a complete stop, then ensure the machine is on level ground or on a robust secure support.

The cutting height may be adjusted from 25mm to 100mm by selecting the appropriate holes in the skids. Always ensure that every setting is the same. Should a wheel kit be fitted then the height is adjustable from 25mm to 300mm.

In both situations remove the bolt, adjust the implement to the desired height, and replace bolt and spring washer then secure nut tightly.

- REMOVING MOWER FROM THE TRACTOR

It is more than likely that you will have the implement in the transport position and therefore the PTO should already be disengaged. Never transport the implement with the PTO engaged. Lower implement and all attachments (i.e. loader buckets) all the way to the ground and wait for all moving parts to come to a complete stop. Turn off tractor ignition and remove key. Firmly apply park break, remove seatbelt and dismount the tractor. Remove check chains (if installed). Disconnect the PTO Shaft from the tractor and implement by de-pressing spring-activated locking pin.

Collapse PTO Shaft inside itself as far as possible and store in a dry area undercover. Block implement securely for storage with robust secure supports then remove top and lower linkage pins. Make sure that the tractor’s lower linkage arms are free and return to tractor seat. Start tractor as specified in the tractors operating manual, select a low gear and slowly drive away from the implement making sure that it does not fall.

- OWNER SERVICE

The information in this section is written for operators who possess basic mechanical skills. Should you need help, your dealer has trained service technicians available. For your protection, read and follow safety information in this manual.

- Disengaged tractor PTO and wait for all moving parts to come to a complete stop. Lower implement to the ground or block with secure robust support. Turn tractor engine off, remove key and disconnect PTO Shaft from the tractor before performing any service or maintenance.

- Keep all persons away from operator control area while performing adjustments, service or maintenance.

- During routine checks and/or repairs ensure that no one can switch on the tractor or Implement accidentally. Keep the key to the tractor in your pocket.

- Always wear relatively tight fitted clothing to avoid entanglement in moving parts. Wear heavy duty, rough-soled boots and protective equipment for eyes, hair, hands, hearing and head.

- BLOCKING METHOD

To minimize the potential hazards of working underneath the implement, these procedures must be followed.

- Never go underneath implements that are lowered to the ground or raised, unless it is properly blocked and secure. Never place any part of the body underneath the implement or between moveable parts even when engine has been turned off. Hydraulic systems can “creep” (i.e. slowly lower). They may fail or movement of the control levers can cause the implement to drop or rotate unexpectedly causing severe injury or death.

- Follow Operator’s Manual instructions for working on your implement, correct blocking procedures or have servicing carried out by a qualified dealer.

- During routine checks and/or repairs ensure that no one can switch on the tractor or Implement accidentally. Keep the key to the tractor in your pocket.

- Jack stands with a load rating of 1000lbs or more are the only approved blocking devices for this implement. A minimum of four (4) jack stands, located under the implement must be installed before working underneath the implement. Do not position jack stands under wheels, axles or wheel supports as components can rotate and cause implement to fall.

- When blocking, you must consider the overall stability of the implement. Just placing jack stands under the unit will not ensure your safety. The working surface must be level and solid to support the weight of the jack stands. Ensure jack stands are stable both top and bottom, and implement is approximately level. With full implement weight lowered onto jack stands, test blocking stability before beginning working underneath implement.

- Implement should be properly and securely attached to the tractor (see operating section), the brakes firmly set, key removed and both front and rear tractor wheels securely blocked.

- LUBRICATION INFORMATION

- Do not let excess grease collect on or around parts, particularly when operating in sandy conditions.

- When greasing implement, be sure to clean fittings thoroughly before attaching grease gun.

- China, in conjunction with the gearbox manufacturer recommends high quality gear oil with a viscosity index of 90W. Remove oil level plug on side of gearbox, fill gearbox until oil runs out the side plug. Refit oil level plug. Check gearbox daily for evidence of leakage. Have your dealer assist you should leakage be found.

- Lubricate the PTO universal joint every eight operating hours. Failure to maintain proper lubrication could result in damage to Universal Joints, gearbox and driveline. To lubricate the shaft itself, lower implement to the ground, disconnect PTO Shaft from tractor and slide the PTO halves out until almost disconnected from each other. Apply a bead of grease completely around male half where it meets the female half. Slide over each other several times repeatedly to distribute grease.

- If the unit is fitted with a roller or rear castor lubricate all grease points every 8 hours.

- CUTTER BAR AND BLADE SERVICING

- It will be necessary to gain access to the bottom side of the Mower to change the blades. Disconnect PTO Shaft from the tractor, raise implement and block securely as outlined in “Blocking Method”, before changing the blades.

- Remove nuts and then carefully remove blade bolt. If blade bolt is seized in the cross bar, to prevent gearbox damage, it may be necessary to completely unbolt rotor before driving the old bolt out.

- If it is necessary to completely remove the rotor it is important that you work from the side of it and not from directly underneath. Also ensure that it is well supported as the cross bars are heavy and could drop suddenly during unbolting, causing severe injury or death.

- Always replace or sharpen both blades at the same time. Inspect blade bolts for nicks and gouges; replace if any are found. Insert blade bolt through lower part of the blade beam, then through the blade with the hardened bush inserted.

- Finally insert through the top part of the blade beam and secure with a nyloc nut. Blade should swivel on the bolt, if it does not, determine the cause (i.e. maybe nut too tight) and rectify.

- Blade beam rotation is anti-clockwise when looking down on the mower. Be sure to install blade cutting edge to lead in an anti-clockwise direction.

- When sharpening blades grind each blade the same amount to maintain balance. Replace blades in pairs. Unbalanced blades will cause excessive vibration that can damage gearbox bearings. Vibration may also cause structural cracks to the implement and/or gearbox housing.

- GEARBOX SERVICE

The information in this manual requires special skills and tools. If you are not properly equipped or your mechanics are not properly trained in this type of repair, you may void your warranty or cause more damage leading to a larger repair bill.

- Never place any part of the body underneath the implement or between moveable parts even when engine has been turned off. Hydraulic systems can “creep” (i.e. slowly lower). They may fail or movement of the control levers can cause the implement to drop or rotate unexpectedly causing severe injury or death.

- Follow Operator’s Manual instructions for working on your implement or have servicing carried out by a qualified dealer.

- During routine checks and/or repairs ensure that no one can switch on the tractor or implement accidentally. Keep the key to the tractor in your pocket.

- Keep all persons away from operator control area while performing adjustments, service or maintenance.

- Always wear relatively tight fitted clothing to avoid entanglement in moving parts. Wear heavy duty, rough-soled boots and protective equipment for eyes, hair, hands, hearing and head.

Fill gearbox with high quality gear oil with a viscosity index of 90W. Fill gearbox until oil runs out the side plug on the gearbox. Gearbox Oil should be changed every 1000 hours of use or every two (2) years. Always drain oil into a container and dispose of it properly to protect the environment. Oil level must be checked daily. The composition of oil deteriorates over time and the intense heat that occurs when operating a gearbox reduces the oil level. Low oil levels will result in wear of gearbox components that overtime will result in component failure that could have been prevented.

- GEARBOX MAINTENANCE

- During routine checks and/or repairs ensure that no one can switch on the tractor or Implement accidentally. Keep the key to the tractor in your pocket.

- Keep all persons away from operator control area while performing adjustments, service or maintenance.

- Do not carry out any servicing on the gearbox if it has been operating. Gearboxes and the oil inside are very hot during operations and for sometime after you have finished and can cause serious burns. Always wait for gearbox to cool down.

Repair to this gearbox should be limited to replacing bearings, seals and gaskets. Replacing gears, shafts and a housing is not cost effective. It is more economical to purchase a complete gearbox if repairs other than bearings, seals and gaskets are required. It is highly recommended that the customer be given the costs involved in both scenarios.

- Inspect gearbox for leakage and bad bearings. Leakage is a very serious problem and must be corrected immediately. Excessive noise and side to side or endplay in the shafts indicate bearing failure. Leakage can occur at the vertical or horizontal gasket and shaft seals. Leakage from the horizontal gasket or seal can be repaired without removing the gearbox from the mower.

- Proper seal installation is important. An improperly installed seal will leak. Always ensure the area where the seal outer diameter sits is clean. Always inspect the area of the shaft where the seal sits and remove any burrs or nicks with emery cloth.

- Always lubricate gear shaft and seal lips before assembling. Remove and replace any seals damaged during assembly.

- When assembling gearboxes always clean housing paying specific attention to areas where gaskets will be installed. Wash housing and all components thoroughly. Select a clean area where you can assemble the gearbox. Replace any damaged seals, bearings or gaskets. All parts must be clean and lightly oiled before reassembled.

- GEARBOX SPARE PARTS

| GEARBOX SPARE PARTS |  |  |  |
| --- | --- | --- | --- |
| Parts | Description | Specification | Qty. |
| 1 | Bolt | M8×25 | 4 |
| 2 | spring washer |  | 4 |
| 3 | shrouding |  | 1 |
| 4 | Input Gear |  | 1 |
| 5 | Bearing |  | 1 |
| 6 | Housing |  | 1 |
| 7 | Hole circlip |  | 1 |
| 8 | Oil Seal |  | 1 |
| 9 | Input Shaft |  | 1 |
| 10 | Bearing |  | 2 |
| 11 | Retaining Ring |  | 1 |
| 12 | Spacer-Bearing |  | 1 |
| 13 | Hole circlip |  | 1 |
| 14 | Oil Seal |  | 1 |
| 15 | Blade Hub |  | 1 |
| 16 | Blade Nut |  | 1 |
| 17 | Cotter Pin | 5×56 | 1 |
| 18 | Shaft-Output |  | 1 |
| 19 | Washer-Blade Hub |  | 1 |
| 20 | Bearing |  | 1 |
| 21 | Hole wire circlip |  | 1 |
| 22 | Dust cover |  | 1 |

- SPARE PARTS FOR SG SERIES MOWER

- SPARE PARTS LIST

| SPARE PARTS LIST |  |  |  |
| --- | --- | --- | --- |
| Parts | Description | Specification | Qty. |
| 1 | Composite Jacket | 30×36×30 | 2 |
| 2 | Bearing Cushion |  | 1 |
| 3 | Spring Washer | 12 | 1 |
| 4 | Bolt | M12×30 | 1 |
| 5 | Bolt | M18×100 | 1 |
| 6 | Tail Wheel Support Arm Weldment |  | 1 |
| 7 | Flat Washer | 18 | 3 |
| 8 | Locknut | M18 | 3 |
| 9 | Shaft Complete, PTO |  | 1 |
| 10 | Chain Fixed Plate |  | 1 |
| 11 | Bolt | M8×20 | 5 |
| 12 | Nut | M16 | 6 |
| 13 | Flat Washer | 16 | 6 |
| 14 | Bolt | M16×50 | 6 |
| 15 | Bolt | M18×170 | 1 |
| 16 | Top Hanger Pin Roll |  | 2 |
| 17 | Flat Washer | 27 | 2 |
| 18 | Lock Pin |  | 4 |
| 19 | Diagonal Member |  | 2 |
| 20 | Middle Rod Weldment |  | 1 |
| 21 | Hanger Spacer Bush |  | 2 |
| 22 | Bolt | 18×150 | 1 |
| 23 | Right Support Plate Weldment |  | 1 |
| 24 | Left Support Plate Weldment |  | 1 |
| 25 | Gear Box |  | 1 |
| 26 | Bolt | M16×45 | 4 |
| 27 | Flat Washer | 16 | 4 |
| 28 | Spring Washer | 16 | 4 |
| 29 | Nut | M16 | 4 |
| 30 | Blade Bar Weldment |  | 1 |
| 31 | Press Plate |  | 1 |
| 32 | Washer |  | 1 |
| 33 | Slotted Hex Nut | M24 | 1 |
| 34 | Cotter Pin | 5×56 | 1 |
| 35 | Lock Nut | M20 | 2 |
| 36 | Rotary Sleeve |  | 2 |
| 37 | Remake Bolt | M20×70 | 2 |
| 38 | Cotter Pin | 5×36 | 2 |
| 39 | Blade |  | 2 |
| 40 | Nut | M22 | 2 |
| 41 | Spring Washer | 22 | 2 |
| 42 | Down Hanger Pin Roll |  | 2 |
| 43 | Frame Weldment |  | 1 |
| 44 | Lock Nut | M14 | 8 |
| 45 | Blot | M14×30 | 8 |
| 46 | Support Frame Weldment |  | 2 |
| 47 | Chain |  |  |
| 48 | Lock Nut | M8 | 7 |
| 49 | Chain Support Beam |  | 1 |
| 50 | Flat Washer | 8 | 2 |
| 51 | Blok | M8×30 | 2 |
| 52 | Tail Wheel Support Weldment |  | 1 |
| 53 | Nut | M20 | 2 |
| 54 | Spring Washer | 20 | 2 |
| 55 | Flat Washer | 20 | 2 |
| 56 | Tail Wheel Spacer Bush |  | 2 |
| 57 | Bearing | 6205-RZ | 2 |
| 58 | Tail Wheel Assembly |  | 1 |
| 59 | Tail Wheel Axis |  | 1 |
| 60 | Flat Washer | 30 | 1 |

- TROUBLE SHOOTING

| Problem | Possible Causes | Solution |
| --- | --- | --- |
| Grass cut is lower in the centre than on the edges. | Height of the mower lower at the front or rear. | Adjust height and attitude so that the front and rear of the mower are within 20mm of the same height. See instructions |
| Mower is “Streaking” (i.e. not cutting cleanly.) | Conditions too wet for cutting. Wet grass. | Allow grass to dry before cutting. |
|  | Blades unable to cut the grass that the tractor tyres pressed down. | Reduce ground speed but maintain maximum rpm at the tractor PTO as specified in Specifications.” Lower cutting height. |
|  | Dull Blades. | Sharpen or replace blades. |
| Material discharged from mower is uneven and in clumps. | Material being cut is too long and/or has too much volume. | Reduce ground speed but maintain maximum rpm at the tractor PTO as specified in “Specifications” or make two passes. First pass higher than the second and the second at 90° to the first. |
|  | Wet grass. | Allow grass to dry before cutting. Reduce ground speed but maintain maximum rpm at the tractor PTO as specified in “Specifications.” |
|  | Rear of mower too low, trapping material underneath. | Adjust mower height and attitude (See instructions.) |

- WARRANTY

All our machinery is covered by a comprehensive warranty against faulty materials and workmanship provided that (i) The machine is operated in normal use, (ii) The machine is correctly serviced and maintained, (iii)The machine is worked within its stated capacity (i.e. Horsepower limit and PTO Speed), (iv) The machine is not subjected to any modifications or damage from improper use, (v)  is in receipt of duly signed pre- delivery and installation certificate and warranty documentation. The term of warranty shall be 12 months from date of installation except machines sold into the Hire Industry; these shall be void of warranty. The following items are not covered by our warranty; (i) Power Take-off shafts. (ii) Blades, knives, tyres, belts etc. (iii) Freight costs, (iv) Loss of income from the machine due to a warranty fault, (v) Use of non genuine parts. The manufacturer’s obligation under this warranty is limited to correcting without charge at its factory or by one of its authorised Dealers, with the manufacturer’s approval, any parts thereof, within 12 months from date of purchase by the original user, and which upon examination shall disclose to the manufacturer’s satisfaction to have been originally defective. Correction of such defects by repair to, or supplying of replacements of defective parts, shall constitute fulfilment of all obligations to the original user: Manufacturer shall not be liable for loss, damage, or expense directly or indirectly from the use of its product or from any other cause. Defective parts must be returned freight paid each way by the purchaser to the manufacturer. This warranty shall apply to any parts which must be replaced because of normal wear, or which have not been repaired or altered outside of the manufacturer’s factory unless authorised by the manufacturer. This warranty should not be construed as a free service period during the warranty period. No person, agent or dealer is authorised to give any warranties on behalf of the manufacturer or to assume for the manufacturer any other liability in connection with any of its product unless made in writing from the manufacturer. All claims will be null and void if any warranty work is commenced without prior approval from us.

- WARRANTY AND SAFETY ACKNOWLEDGEMENT

To be completed by the purchaser at the time of delivery and returned to us

Owners Name and Address:

……………………………………………………………………………………………………………………………………………………………………………………

I hereby acknowledge:

1. I have received and accepted delivery of the machine listed above.

2. The machine has been delivered to me with all systems functioning and running to my satisfaction.

3. I have been instructed and understand the application, limitations and capabilities that the machine has been designed for.

4. I understand and accept the warranty and understand I am responsible to pay for travelling costs and expendable items.

5. I have received an operator manual and have reviewed their content regarding the correct operation of the machine.

6. The dealer has supplied an owner’s manual with the machine and reviewed the sections on safety with me making any explanations that may be required.

7. I am particularly aware of the importance of not operating the machine outside the safety guidelines.

8. I understand the principles of safe operation of this machine and accept responsibility to ensure that any persons who operate this machine are properly instructed in these matters.

9. In the event that the regulatory authorities or we should require modifications or new service procedures to be made or adopted we indemnify the dealer from any liability or cost which may be involved.

10. In the event that ownership of this machine should pass from me, I undertake to pass responsibility for the above matters on to the new owner and I will advise the change of ownership.